### 欧盟高度关注物质SVHC候选清单

自2008年10月28日，欧盟化学品管理局首次公布第一批SVHC候选清单，截至2012年12月19日，SVHC候选清单物质已更新8次。目前正式公布的候选清单中包含138项SVHC物质，包括多种无机物和有机物，应用遍布各个行业。

□ SVHC第一批清单（15项）于2008年10月28日正式公布生效

□ SVHC第二批清单（13项）于2010年1月13日正式公布生效，2012年6月18日，ECHA将第二批SVHC清单中索引号为650-017-00-8的硅酸铝耐火陶瓷纤维（Al-RCF）和氧化锆硅酸铝耐火陶瓷纤维（ZrAl-RCF）整合到第六批SVHC清单中范围更广的Al-RCF和ZrAl-RCF条目下，由此第二批SVHC清单由15项减少为13项。

□ 2010年3月30日，丙烯酰胺被列入SVHC清单(属第二批清单）

□ SVHC第三批清单（8项）于2010年6月18日正式公布生效

□ SVHC第四批清单（8项）于2010年12月15日正式公布生效

□ SVHC第五批清单（7项）于2011年6月20日正式公布生效

□ SVHC第六批清单（20项）于2011年12月19日正式公布生效

□SVHC第七批清单（13项）于2012年6月18日正式公布生效

□SVHC第八批清单（54项）于2012年12月19日正式公布生效

Table 1 : The 84 SVHCs officially published by the ECHA

表格1: 欧洲化学品管理局正式公布的84种高度关注物质

| 项目 | Substance Name 物质名 | CAS number CAS号码 | EC number EC号码 | Possible Applications 可能的应用 |
| --- | --- | --- | --- | --- |
| 1 | 4,4'- Diaminodiphenylmethane 4,4’-二氨基二苯甲烷 | 101-77-9 | 202-974-4 | - Hardener for epoxy resins and adhesives, intermediate in manufacture of high performance polymers, raw material for azodyes manufacturing. 用於环氧树脂固化剂和胶粘剂，生产高性能聚合物的中间体，制造偶氮染料的原料 |
| 2 | 5-tert-butyl-2,4,6-trinitro-m-xylene (musk xylene) 二甲苯麝香 | 81-15-2 | 201-329-4 | - Fragrance ingredient 香精中的人造麝香 |
| 3 | Alkanes, C10-13, chloro (Short Chain Chlorinated Paraffins) 短链氯化石蜡 (C10-13) | 85535-84-8 | 287-476-5 | - Flame retardant in textiles, rubber, used in paints, sealant, adhesives 纺织品及橡用胶阻燃，用於塗料、密封剂、粘合剂 |
| 4 | Anthracene 蒽 | 120-12-7 | 204-371-1 | - Manufacture of pyrotechnic products, catalyst in wood pulp production, raw material for anthracene oil manufacturing, base material for anthraquinone dyes. 制造烟火产品，生产木浆的催化剂，生产蒽油的原材料，生产蒽醌染料的基本材料 |
| 5 | Benzyl butyl phthalate (BBP) 邻苯二甲酸丁苄酯 | 85-68-7 | 201-622-7 | - Plasticiser for PVC, synthetic leather. Plastisol / flock printing compounds. 聚氯乙烯增塑剂，人造橡胶，人造皮革 |
| 6 | Bis (2-ethyl(hexyl)phthalate) (DEHP) 邻苯二甲酸二(2-乙基己基)酯 （2-乙基己酯） | 117-81-7 | 204-211-0 | - Plasticiser for PVC, synthetic leather. Plastisol / flock printing compounds. 聚氯乙烯的增塑剂，人造橡胶，人造皮革 |
| 7 | Bis(tributyltin)oxide (TBTO) 氧化双三丁基錫 | 56-35-9 | 200-268-0 | - Antimicrobial agents, biocidal products, polyurethane foam manufacturing, polymer products for flooring, tiles, carpeting, textiles for upholstery, treatment of feather and down. 抗菌剂，生物农药产品，聚氨酯泡沫生产， 地板、瓷砖、地毯产品用聚合物 ，傢俱用纺织品，羽绒处理 |
| 8 | Cobalt dichloride 氯化钴 | 7646-79-9 | 231-589-4 | - Silica gel indicator, dye mordant in glass industry, recycling scrap metal, invisible ink, drying agent in paints, lacquers, varnishes, printing inks, electroplating, additive in tyres. 防潮珠，玻璃行业染料染剂，废金属再造，隐形油墨、油漆、清漆、油漆及油墨的乾燥剂，电镀，轮胎的添加剂 |
| 9 | Diarsenic pentaoxide 五氧化二砷 | 1303-28-2 | 215-116-9 | - Manufacture of wood preservative, dying, manufacturing of special glass. 制造木材的防腐剂，染色，生产特种玻璃 |
| 10 | Diarsenic trioxide 三氧化二砷 | 1327-53-3 | 215-481-4 | - Use in manufacturing of glass, enamels and lead crystals, raw material for arsenic alloys, arsenide semiconductors, wood preservative. 用於制造玻璃、搪瓷和铅晶体，砷合金原料，砷化镓半导体，木材防腐剂 |
| 11 | Dibutyl phthalate (DBP) 邻苯二甲酸二丁基酯 | 84-74-2 | 201-557-4 | - Plasticiser for PVC, synthetic leather. Plastisol / flock printing compounds. 聚氯乙烯增塑剂，人造橡胶，人造革 |
| 12 | Hexabromocyclododecane (HBCDD) and all major diastereoisomers identified (α – HBCDD, β-HBCDD, γ-HBCDD) 六溴环十二烷 | 25637-99-4 3194-55-6 (134237-50-6, 134237-51-7, 134237-52-8) | 247-148-4 221-695-9 | - Flame retardant, expandable polystyrene EPS (packaging material), extruded polystyrene XPS (insulation board in construction), high impact polystyrene HIPS (housing for E&E products). 阻燃剂，可发性聚苯乙烯EPS（包装材料），挤塑聚苯乙烯XPS（建築用保温板），高抗衝性聚苯乙烯高HIPS（房屋及电子产品用） |
| 13 | Lead hydrogen arsenate 砷酸氢铅 | 7784-40-9 | 232-064-2 | - Wood preservative 木材防腐剂 |
| 14 | Sodium dichromate 重铬酸钠二水合物 | 7789-12-0 10588-01-9 | 234-190-3 | - Agent used in dyeing, mordant agent, manufacture of chromium pigments, preparation of coloured glass and ceramic glazes. 用於印染，媒染剂，铬颜料制造，有色玻璃和陶瓷釉料的準备 |
| 15 | Triethyl arsenate 三乙基砷酸酯 | 15606-95-8 | 427-700-2 | - Preservative in wood, used in textiles. 木材防腐剂，纺织品 |
| 16 | Anthracene oil 蒽油 | 90640-80-5 | 292-602-7 | - Mainly used as an intermediate in the production of anthracene, which is used in the production of artificial dyes 主要用作生产蒽的中间体 (蒽是用于生产人造染料) Other applications include: 其它用途包括： - technical tar oils, paints 技术用沥青油、涂料 - pharmaceutical products 药剂制品 - Impregnation agent 浸透剂 - Waterproof membranes for roofing and other sealing purposes 盖屋顶及其它作密封用途的防水膜 - Industrial viscosity modifier 工业用粘度改良添加剂 |
| 17 | Anthracene oil, anthracene paste 蒽油，蒽糊 | 90640-81-6 | 292-603-2 |  |
| 18 | Anthracene oil, anthracene paste, anthracene fraction 蒽油，蒽糊，蒽馏分 | 91995-15-2 | 295-275-9 |  |
| 19 | Anthracene oil, anthracene paste, distn. Lights 蒽油，蒽糊，轻油 | 91995-17-4 | 295-278-5 |  |
| 20 | Anthracene oil, anthracene-low 蒽油，含蒽量少 | 90640-82-7 | 292-604-8 |  |
| 21 | Pitch, coal tar, high temp. (CTPHT) 煤沥青，高温 | 65996-93-2 | 266-028-2 | - Binding agent for (industrial) electrodes and refractories, briquettes (工业用) 电极及耐火煤砖用粘接剂 - Anti-corrosion protection agent in (industrial) paints and coatings (工业用) 涂料及涂层防锈保护剂 - Production of carbon black, pyrene, active carbon and carbon fibres 生产碳黑、嵌二萘(芘) 、活性炭、碳纤维 - Waterproof coating/ adhesive agent in roofing 防水涂层或盖屋顶用粘合剂 Products that may contain CTPHT: 可能含有CTPHT的产品包括： - Clay pigeons 泥鸽 - Medicinal preparations 药剂 - waterproof coating/adhesive agent in roofing and heavy duty corrosion protection products 防水涂层或盖屋顶用粘合剂及重型防锈保护产品 |
| 22 | 2,4-Dinitrotoluene (DNT) 2,4-二硝基甲苯 | 121-14-2 | 204-450-0 | - An intermediate used during the production of flexible polyurethane foams 在生产软质聚氨酯泡沫塑料中的中间体 - Synthesis of azodyes 偶氮染料生产 - An intermediate in the manufacturing of rubber chemicals and automotive airbags - 在生产橡胶化学物品及汽车安全气袋中的中间体 - A gelatinizing-plasticizing agent in explosives 爆炸品内的胶凝-塑化剂 |
| 23 | Diisobutyl phthalate (DIBP) 邻苯二甲酸二异丁酯 | 84-69-5 | 201-553-2 | - A plasticizer. Sometimes used as a substitute for dibutylphthalate (DBP) 增塑剂。有时用作邻苯二甲酸二丁酯(DBP) 代替物 - May be present in: plastics, paints, lacquers, varnishes, paper, pulp and board, adhesives, binding agents, softeners, viscosity adjusters, dispersion glues and printing inks for paper and packaging 可含于：塑胶、油漆、颜料、涂料、清漆、纸，纸浆及纸板、粘接剂、粘合剂、柔软剂、粘度调整剂、分散性胶水、纸制品油墨 |
| 24 | Lead chromate 铬酸铅 (铬黄) | 7758-97-6 | 231-846-0 | - Lead chromate based paints are found in all industrial sectors in products requiring protection against corrosion or bright colours. It also has anticorrosive property for primary coating 以铬黄为主的涂料可以在各个有需要预防腐蚀的工业用途上找到。其防腐蚀性质也有用于底涂 |
| 25 | Lead chromate molybdate sulfate red (C.I. Pigment Red 104) 钼铬酸铅红 (C.I. 颜料红104) | 12656-85-8 | 235-759-9 | - Lead chromate based pigments 以铬黄为主的涂料 - Applications include: colouring of plastics (polyolefins, PVC, nylon), inks, paints, varnishes, coatings, paper, textile printing, leather finishing, linoleum, mastics 用途包括：塑胶染色(聚烯烃、聚氯乙烯、尼龙) 、油墨、清漆、涂层、纸、纺织印刷、皮革涂饰、油地毡、胶浆 |
| 26 | Lead sulfochromate yellow (C.I. Pigment Yellow 34) 铬酸铅黄 (C.I. 颜料黄34) | 1344-37-2 | 215-693-7 |  |
| 27 | Tris (2-chloroethyl) phosphate (TCEP) 三(2-氯乙基) 磷酸酯 | 115-96-8 | 204-118-5 | A flame retardant and plasticiser. Applications include: 主要用途为阻燃剂和增塑剂。其他用途包括： - Rubber and plastic products 橡胶及塑胶产品 - Adhesives, coatings 粘合剂、涂层 - Binding agents 粘接剂 - Flame retardants and extinguishing agents 阻燃剂及灭火剂 - Furniture (polyurethane foam, upholstery coating) 家具(聚氨酯泡沫、装饰用涂层) - Insulating materials 绝缘材料 |
| 28 | Acrylamide 丙烯酰胺 | 79-06-1 | 201-173-7 | - Mainly used in the manufacture of polyacrylamide. May be used as a grouting agent in specialist applications. Other possible applications include: additive in cosmetics, soil and sand stabilisation, coatings and paints, textile processing 主要用途为生产聚丙烯酰胺。在专业用途可能用作灌浆材料。其他用途包括：化妆品添加剂、沙土稳定、涂层及涂料、纺织品处理 |
| 29 | Trichloroethylene 三氯乙烯 | 79-01-6 | 201-167-4 | - Cleaning and degreasing of metal parts 金属部件清洁与脱脂 - Solvent in adhesives 粘合剂溶剂 - Intermediate in the manufacture of chlorinated and fluorinated organic compounds 含氯和含氟有机化合物生产中的中间体 |
| 30 | Boric acid 硼酸 | 10043-35-3 11113-50-1 | 233-139-2 234-343-4 | - Uses include a multitude of applications, e.g. in biocides and preservatives, food additives, glass, ceramics, adhesives, fertilisers, flame retardants, paints, industrial fluids, brake fluids, soldering products, film developers 应用广泛，包括：杀虫剂、防腐剂、化妆品、食品添加剂、玻璃、陶瓷、粘合剂、化肥、阻燃剂、涂料、工业液体、制动液、焊锡产品、显影剂 |
| 31 | Disodium tetraborate, anhydrous 四硼酸钠 (无水) | 1330-43-4 12179-04-3 1303-96-4 | 215-540-4 | - Uses include a multitude of applications, e.g. in glass and glass fibres, ceramics, detergents and cleaners, personal care products, industrial fluids, metallurgy, adhesives, flame retardants, biocides, fertilizers 应用广泛，包括：玻璃和玻璃纤维、陶瓷、洗涤剂和清洁剂、个人护理用品、工业液体、冶金、添加剂、阻燃剂、生物杀灭剂、化肥 |
| 32 | Tetraboron disodium heptaoxide, hydrate 水合硼酸钠 | 12267-73-1 | 235-541-3 | - Uses include a multitude of applications, e.g. in glass and glass fibres, ceramics, detergents and cleaners, personal care products, industrial fluids, metallurgy, adhesives, flame retardants, biocides, fertilizers 应用广泛，包括：玻璃和玻璃纤维、陶瓷、洗涤剂和清洁剂、个人护理用品、工业液体、冶金、添加剂、阻燃剂、生物杀灭剂、化肥 |
| 33 | Sodium chromate 铬酸钠 | 7775-11-3 | 231-889-5 | - Laboratory (analytical agent) 实验室（分析试剂） - Manufacture of other chromium compounds 其它铬酸盐化合物生产 |
| 34 | Potassium chromate 铬酸钾 | 7789-00-6 | 232-140-5 | - Corrosion inhibitor for treatment and coating of metals 金属表面处理和用作涂层内的抗蚀剂 - Manufacture of reagents and chemicals 试剂与化学品生产 - Manufacture of textiles 纺织品制造 - Colouring agent in ceramics 陶瓷染色剂 - Manufacture of pigments/inks 色素和墨水生产 - Laboratory (analytical reagent) 实验室（分析试剂） |
| 35 | Ammonium dichromate 重铬酸铵 | 7789-09-5 | 232-143-1 | - Oxidising agent 氧化剂 - Laboratory (analytical agent) 实验室（分析试剂） - Textile mordant 纺织品媒染剂 - Manufacture of photosensitive screens 感光屏幕制造 - Metal treatment 金属表面处理 |
| 36 | Potassium dichromate 重铬酸钾 | 7778-50-9 | 231-906-6 | - Chrome metal manufacturing 金属制造 - Corrosion inhibitor for treatment and coating of metals 金属表面处理和用作涂层内的抗蚀剂 - Manufacture of reagents and chemicals 试剂与化学品生产 - Laboratory (analytical agent) 实验室（分析试剂） - Cleaning of laboratory glassware 实验室玻璃器皿清洗 - Textile mordant 纺织品媒染剂 - Oxidising agent in photolithography 照相平版用氧化剂 |
| 37 | Cobalt(II) sulphate 硫酸钴 | 10124-43-3 | 233-334-2 | - Production of chemicals, pigments, catalysts and driers, surface treatments (such as electroplating), corrosion prevention, decolourising (in glass, pottery) and batteries. 用于化学品， 颜料，催化剂和干燥剂的生 产,表面处理（如电镀），防锈，（玻璃，陶 瓷）脱色剂和电池等物质的生产。 |
| 38 | Cobalt(II) dinitrate 硝酸钴 | 10141-05-6 | 233-402-1 | - Production of other chemicals including catalysts. Further applications may include surface treatment and batteries. 其它化学品包括催化剂的生产。 进一步的用途可包括表面处理剂以及电池 中的应用 |
| 39 | Cobalt(II) carbonate 碳酸钴 | 513-79-1 | 208-169-4 | - Mainly used in the manufacture of catalysts. 主要用于催化剂的制备。 - Minor uses may include production of other chemicals including pigments, and as an adhesive in ground coat frit. 其它用途包括化学品如颜料的生产以及附 着材料（地面釉料）。 |
| 40 | Cobalt(II) diacetate 醋酸钴 | 71-48-7 | 200-755-8 | - Mainly used in the manufacture of catalysts or as a catalyst. 主要用于催化剂的制备。 - Minor uses may include production of other chemicals including pigments, surface treatment in alloys, dyes and rubber adhesion. 其次用于其它化学品包括颜料的生产,表面 处理剂、合金、染料以及橡胶附着剂. |
| 41 | 2-Methoxyethanol 乙二醇单甲醚 | 109-86-4 | 203-713-7 | - Uses include as a solvent, a chemical intermediate or a laboratory chemical. 用途包括作溶剂、化学中间体以及实验室化学品。 |
| 42 | 2-Ethoxyethanol 乙二醇单乙醚 | 110-80-5 | 203-804-1 | - Uses include as a solvent, a chemical intermediate or a laboratory chemical. 用途包括作溶剂、化学中间体以及实验室化学品。 |
| 43 | Chromium trioxide 三氧化铬 | 1333-82-0 | 215-607-8 | - Mainly used in metal finishing, such as electroplating (e.g. hard chrome and decorative plating), conversion coatings and brightening. It is also used as a fixing agent in waterborne wood preservatives. Minor uses are in the manufacture of the pigments and paints, in catalyst and detergent manufacture, and as and oxidising agent. 主要用于金属加工，例如电镀（硬铬和装饰电镀），转化涂层和打磨。也应用于防水木的固定剂。其次用于颜料和涂料，催化剂和洗涤剂的制备，以及作为氧化剂。 |
| 44 | Acids generated from chromium trioxide and their oligomers: 三氧化铬及其它低聚物所生成的酸类： Chromic acid 铬酸 Dichromic acid 重铬酸 Oligomers of chromic acid and dichromic acid 铬酸和重铬酸的低聚物 | 7738-94-5 13530-68-2 | 231-801-5 236-881-5 | - These acids and their oligomers are generated when chromium trioxide is dissolved in water. Consequently, the uses of these substances are the same as indicated for chromium trioxide. 当三氧化铬溶于水时，可形成这些酸及其低聚物。因此，这些物质的用途常以三氧化铬来表示。 |
| 45 | 2-ethoxyethyl acetate 乙二醇乙醚乙酸酯 | 111-15-9 | 203-839-2 | - Solvent in coatings and in the chemical industry 涂料和化工的溶剂 - As an intermediate in the manufacture of cyanoacrylate adhesives 作为氰基丙烯酸酯粘合剂生产的中间体 |
| 46 | strontium chromate 铬酸锶 | 7789-06-2 | 232-142-6 | - Mainly as corrosion inhibitor in coating mixtures used in the aeronautic/aerospace sector, in the coil coating sector of steel and aluminum, and in the vehicle coating sector. 主要用于涂料的抗蚀剂应用于航空航天，钢/铝制品的卷材及车辆 |
| 47 | 1,2-Benzenedicarboxylic acid, di-C7-11-branched and linear alkyl esters (DHNUP) 1,2 - 苯二酸 - 二（C7-11支链与直链）烷基酯（DHNUP） | 68515-42-4 | 271-084-6 | - Plasticiser in PVC, foam, adhesives and coatings. 聚氯乙烯、泡沫、黏合剂和涂料的增塑剂 |
| 48 | hydrazine 肼 | 7803-57-8; 302-01-2 | 206-114-9 | - As an intermediate in the manufacture of hydrazine derivatives 作为中间体应用于肼系的衍生物的生产 - As a monomer in polymerizations 作为聚合反应的单体 - Corrosion inhibitor in water treatment 水处理的抗蚀剂 - Metal reduction and refining of chemicals 金属还原和化学试剂提纯 |
| 49 | 1-methyl-2- pyrrolidone 1-甲基-2-吡咯烷酮 | 872-50-4 | 212-828-1 | - Solvent in coatings and cleaning products, for the EEE manufacture, semiconductor industry, petrochemical processing, pharmaceuticals and agrochemicals. 涂料, 清洁产品，电子电气设备制造，半导体工业，石油化学加工，医药品和农业化学品的溶剂 |
| 50 | 1,2,3-trichloropropane 1,2,3-三氯丙烷 | 96-18-4 | 202-486-1 | - As an intermediate in the manufacture of chlorinated solvents and agricultural products 作为中间体用于氯化溶剂和农产品的生产 - As a monomer in polymerisations 作为聚合的单体 - Solvent, paint and varnish remover 溶剂，油漆和清漆的去除剂 - Degreasing agent 脱脂剂 |
| 51 | 1,2-Benzenedicarboxylic acid, di-C6-8-branched alkyl esters, C7-rich (DIHP) 邻苯二甲酸二（C6-8支链）烷基酯，富C7（DIHP） | 71888-89-6 | 276-158-1 | - Plasticiser in PVC and in sealants, coatings Printing inks PVC、密封剂、涂料和印刷油墨的增塑剂 |
| 52 | 1,2-dichloroethane 1,2-二氯乙烷 | 107-06-2 |  | - 1,2-dichloroethane is mainly used for manufacture of other substances. Minor uses as solvent in the chemical and pharmaceutical industry, as well as in laboratories. 主要用于制造其它物质，次要用途于化学和制药工业中用作溶剂。 |
| 53 | 2,2'-dichloro-4,4'-methylenedianiline 2,2'-二氯-4,4'-二氨基二苯基甲烷 | 101-14-4 |  | - 2,2'-dichloro-4,4'-methylenedianiline is mainly used as curing agent in resins and in the production of polymer articles and also for manufacture of other substances. The substance may further be used in construction and arts. 作为树脂固化剂，生产聚合物及其他物质。该物质也可能用于建筑和艺术品。 |
| 54 | 2-Methoxyaniline; o-Anisidine 2-甲氧基苯胺，邻氨基苯甲醚 | 90-04-0 |  | - 2-Methoxyaniline is mainly used in the manufacture of dyes for tattooing and coloration of paper, polymers and aluminium foil. 主要用于制造纹身材料和纸，聚合物和铝箔的着色。 |
| 55 | 4-(1,1,3,3-tetramethylbutyl)phenol 4-(1,1,3,3-四甲基丁基)苯酚 | 140-66-9 |  | - 4-(1,1,3,3-tetramethylbutyl)phenol is mainly used in the manufacture of polymer preparations and of ethoxylate surfactants. It is further used as a component in adhesives, coatings, inks and rubber articles. 主要用于制造聚合物和聚氧乙烯醚。还用作粘合剂、涂料、油墨和橡胶制品的组成部分。 |
| 56 | Aluminosilicate Refractory Ceramic Fibres 硅酸铝耐火陶瓷纤维 are fibres covered by index number 650-017-00-8 in Annex VI, part 3, table 3.1 of Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures, and fulfil the three following conditions: a) oxides of aluminium and silicon are the main components present (in the fibres) within variable concentration ranges b) fibres have a length weighted geometric mean diameter less two standard geometric errors of 6 or less micrometres (µm) c) alkaline oxide and alkali earth oxide (Na2O+K2O+CaO+MgO+BaO) content less or equal to 18% by weight | --- |  | - Refractory ceramic fibres are used for high-temperature insulation, almost exclusively in industrial applications (insulation of industrial furnaces and equipment, equipment for the automotive and aircraft/aerospace industry) and in fire protection (buildings and industrial process equipment). 高温隔热耐火陶瓷纤维，几乎完全用于工业（包括为工业炉窑设备、汽车和飞机/航空航天设备的绝缘）和防火（包括建筑和工业加工设备）。 |
| 57 | Arsenic acid 砷酸 | 7778-39-4 |  | - Arsenic acid is mainly used to remove gas bubbles from ceramic glass melt (fining agent) and in the production of laminated printed circuit boards. To lesser extent the substance is also used in the manufacture of semiconductors and as laboratory agent. 主要用于从熔融陶瓷玻璃中去除气泡，生产夹层印刷电路板时提高铜箔和树脂之间的附着力。其次该物质也有用于生产半导体和用作实验溶剂。 |
| 58 | Bis(2-methoxyethyl) ether 二乙二醇二甲醚 | 111-96-6 |  | - Bis(2-methoxyethyl) ether is used primarily as a reaction solvent or process chemical in a wide variety of applications. It is also used as solvent for battery electrolytes, and possibly in other products such as sealants, adhesives, fuels and automotive care products. 主要作为反应溶剂，也用作电池电解液的溶剂，并可能在其它产品，如密封剂，胶粘剂，燃料和汽车护理产品中作为溶剂使用。 |
| 59 | Bis(2-methoxyethyl) phthalate 邻苯二甲酸二甲氧乙酯 | 117-82-8 |  | - Main uses in the past were as plasticiser in polymeric materials and paints, lacquers and varnishes, including printing inks. 主要用途是作为高分子材料、涂料、油漆和清漆（包括印刷油墨）中的增塑剂。 |
| 60 | Calcium arsenate 砷酸钙 | 7778-44-1 |  | - Calcium arsenate is present in complex raw materials (which themselves are by-products from metallurgical processes) that are used mainly for copper and lead refining. The substance is used to precipitate nickel from the molten metal and to manufacture diarsenic trioxide. However, most of the substance seems to be disposed of as waste. 目前，砷酸钙存在于生产铜、铅和贵金属的复杂的进口原料中。主要用于从熔融金属中提取出镍，制造三氧化二砷等冶金提炼过程。而大部分作为废弃物处置。 |
| 61 | Dichromium tris(chromate) 铬酸铬 | 24613-89-6 |  | - Dichromium tris(chromate) is mainly used in mixtures for metal surface treatment in the aeronautic/aerospace, steel and aluminium coating sectors. 主要用于航空/航天、钢铁和铝涂料行业中的金属表面处理。 |
| 62 | Formaldehyde, oligomeric reaction products with aniline 甲醛与苯胺的低聚反应产物 | 25214-70-4 |  | - Technical MDA is mainly used for manufacture of other substances. Minor uses are as ion exchange resins in nuclear power plants, as hardener for epoxy resins, e.g. for the production of rolls, pipes and moulds, and as well for adhesives. 主要用于制造其它物质。次要用途是在核电厂作为离子交换树脂，环氧树脂固化剂，例如生产卷丶管道和模具，以及粘合剂。 |
| 63 | Lead diazide, Lead azide 叠氮化铅 | 13424-46-9 |  | - Lead diazide is mainly used as initiator or booster in detonators for both civilian and military uses and as initiator in pyrotechnic devices. 主要用作民用和军事用途的雷管和烟火装置的启动器或增压器。 |
| 64 | Lead dipicrate 苦味酸铅 | 6477-64-1 |  | - The substance is an explosive like lead diazide and lead styphnate. It may be used in low amounts in detonator mixtures together with the two other mentioned lead compounds. 苦味酸铅如迭氮化铅和收敛酸铅一样有爆炸性。少量該物质會与其它两种铅化合物混合用于雷管中。 |
| 65 | Lead styphnate 收敛酸铅 | 15245-44-0 |  | - Lead styphnate is mainly used as a primer for small calibre and rifle ammunition. Other common uses are in ammunition pyrotechnics, powder actuated devices and detonators for civilian use. 主要用于小口径步枪弹药底漆。其它常见的用途是烟火弹药，驱动装置和民用雷管。 |
| 66 | N,N-dimethylacetamide N,N-二甲基乙酰胺 | 127-19-5 |  | - N,N-dimethylacetamide is used as solvent, mainly in the manufacture of various substances and in the production of fibres for clothing and other applications. Also used as reagent, and in products such as industrial coatings, insulation paper, polyimide films, paint strippers and ink removers. - 用于制造各种物质和在生产衣服纤维和其他应用過程中作为溶剂使用，也用作去除工业涂料、聚酰亚胺薄膜、脱漆剂和油墨等的溶剂。 |
| 67 | Pentazinc chromate octahydroxide 锌黄 | 49663-84-5 |  | - Pentazinc chromate octahydroxide is mainly used in coatings in the vehicle coating and aeronautic / aerospace sectors. 主要用于航空/航天用涂料和汽车涂料。 |
| 68 | Phenolphthalein 酚酞 | 77-09-8 |  | - Phenolphthalein is mainly used as laboratory agent (pH indicator solutions). Minor uses are in pharmaceutical preparations and in some special applications (e.g. pH-indicator paper, disappearing inks). 主要用作实验室试剂（pH值指标溶液），其次用于生产药用产品和一些特別的用品例如pH试纸和隐形墨水。 |
| 69 | Potassium hydroxyoctaoxodizincate-dichromate 氢氧化铬酸锌钾 | 11103-86-9 |  | - Potassium hydroxyoctaoxodizincatedichromate is mainly used in coatings in the aeronautic/ aerospace, steel and aluminium coil coating and vehicle coating sectors. 主要用于航空/航天用涂料，钢铁和铝卷材涂料和汽车涂料行业。 |
| 70 | Trilead diarsenate 砷酸铅 | 3687-31-8 |  | - Trilead diarsenate is present in complex raw materials for manufacture of copper, lead and a range of precious metals. The trilead diarsenate contained in the raw materials is in the metallurgical refinement process transformed to calcium arsenate and diarsenic trioxide; whereas most of the calcium arsenate appears to be disposed of as waste the diarsenic trioxide is used further. 目前，砷酸铅存在于生产铜丶铅和贵金属的复杂的进口原料中。在原料中所含的砷酸铅在冶金细化过程中转化砷酸钙和三氧化二砷。大部分的钙砷酸作为处置废物而三氧化二砷则可继续使用。 |
| 71 | Zirconia Aluminosilicate Refractory Ceramic Fibres 氧化锆硅铝酸耐火陶瓷纤维 are fibres covered by index number 650-017-00-8 in Annex VI, part 3, table 3.1 of Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures, and fulfil the three following conditions: a) oxides of aluminium, silicon and zirconium are the main components present (in the fibres) within variable concentration ranges b) fibres have a length weighted geometric mean diameter less two standard geometric errors of 6 or less micrometres (µm). c) alkaline oxide and alkali earth oxide (Na2O+K2O+CaO+MgO+BaO) content less or equal to 18% by weight | --- |  | - Refractory ceramic fibres are used for high-temperature insulation, almost exclusively in industrial applications (insulation of industrial furnaces and equipment, equipment for the automotive and aircraft/aerospace industry) and in fire protection (buildings and industrial process equipment). 高温隔热耐火陶瓷纤维，几乎完全用于工业（包括为工业炉窑设备丶汽车和飞机/航空航天设备的绝缘）和防火（包括建筑和工业加工设备）。 |
| 72 | 1,2-bis (2-methoxyethoxy)ethane (TEGDME; triglyme) 三甘醇二甲醚 | 112-49-2 | 203-977-3 | - Mainly used as a solvent or as a processing aid in the manufacture and formulation of industrial chemicals. 主要用作工业化学品生产及配制过程中的溶剂及加工助剂； - Minor uses in brake fluids and repair of motor vehicles. 小部分用于制动液及机动车维修。 |
| 73 | 1,2-dimethoxyethane; ethylene glycol dimethyl ether (EGDME) 乙二醇二甲醚 | 110-71-4 | 203-794-9 | - Mainly used as a solvent or as a processing aid in the manufacture and formulation of industrial chemicals, including use as an electrolyte solvent in lithium batteries. 主要用作工业化学品生产及配制过程中的溶剂及加工助剂以及锂电池的电解质溶液。 |
| 74 | Diboron trioxide 三氧化二硼 | 1303-86-2 | 215-125-8 | - Used in a multitude of applications, e.g. in glass and glass fibres, frits, ceramics, flame retardants, catalysts, industrial fluids, metallurgy, electrical equipment, adhesives, inks/paints, film developing solutions, detergents and cleaners, reagent chemicals, biocides and insecticides. 被应用于众多范畴，如玻璃及玻璃纤维，釉料，陶瓷，阻燃剂， 催化剂 ，工业流体，冶金，电子设备，粘合剂，油墨及油漆，显影剂，清洁剂，生物杀虫剂等。 |
| 75 | Formamide 甲酰胺 | 75-12-7 | 200-842-0 | - Mainly used as an intermediate in the manufacture of agrochemicals, pharmaceuticals and industrial chemicals. 主要用于制造农用化学品，医药品及工业化学品的中间体。 - Minor uses as a solvent, as a laboratory reagent for quality control purposes in forensic laboratories, hospitals, pharmaceutical companies, food and drinks manufacturers and research laboratories. The substance seems to also be used as a plasticiser. 小部分用作溶剂，作为实验室试剂为法医实验室、医院、制药公司、食品和饮料制造商和研究实验室作质量控制。这种物质也被用作增塑剂。 |
| 76 | Lead(II) bis(methanesulfonate) 甲基磺酸铅 | 17570-76-2 | 401-750-5 | - Mainly used in plating processes (both electrolytic and electroless) for electronic components (such as printed circuit boards). The substance seems to also be used for batteries in special applications. 主要用作电子元器件 (例如印刷电路板)的电镀及化学镀的镀层。这种物质也被用于某些特殊用途的电池。 |
| 77 | 1,3,5-tris(oxiranylmethyl)-1,3,5-triazine-2,4,6(1H,3H,5H)-trione (TGIC) 异氰尿酸三缩水甘油酯 | 2451-62-9 | 219-514-3 | - Mainly used as a hardener in resins and coatings. Also used in inks for the printed circuit board industry, electrical insulation material, resin moulding systems, laminated sheeting, silk screen printing coatings, tools, adhesives, lining materials and stabilisers for plastics. 主要用于树脂及涂料固化剂、电路板印刷业的油墨、电器绝缘材料、树脂成型系统、薄膜层、丝网印刷涂料、模具、粘合剂、纺织材料、塑料稳定剂。 |
| 78 | 1,3,5-tris[(2S and 2R)-2,3-epoxypropyl]-1,3,5-triazine-2,4,6-(1H,3H,5H)-trione (β-TGIC) 替罗昔隆 | 59653-74-6 | 423-400-0 | - Mainly used as a solder mask ink in the EU. Also used in electrical insulation material, resin moulding systems, laminated sheeting, silk screen printing, coatings, tools, adhesives, lining materials and stabilisers for plastics. 在欧盟市场上主要用于阻焊油墨。也被用于电器绝缘材料、树脂成型系统、薄膜层、丝网印刷涂料、模具、粘合剂、纺织材料、塑料稳定剂。 |
| 79 | 4,4'-bis(dimethylamino)benzophenone (Michler's ketone) 4,4'-二(N,N-二甲氨基)二苯甲酮(米氏酮) | 90-94-8 | 202-027-5 | - Used as an intermediate in the manufacture of triphenylmethane dyes and other substances. 用于三苯（基）甲烷染料及其他物质制造 的反应中间物 。 - Further potential uses include use as an additive (photosensitiser) in dyes and pigments, in dry film products and as a process chemical in the production of electronic circuit boards. 未来有可能作为染料、颜料及光阻干膜产 品的添加剂（感光剂）、制作电子线路板 的制版化学品。 |
| 80 | N,N,N',N'-tetramethyl-4,4'-methylenedianiline (Michler's base) 4,4’-(对二甲氨基)二苯基甲烷 (米氏碱) | 101-61-1 |  | - Used as an intermediate in the manufacture of dyes and other substances. 用作制造染料及其他物质的反应中间物 。 |
| 81 | [4-[[4-anilino-1-naphthyl][4-(dimethylamino)phenyl]methylene]cyclohexa-2,5-dien-1-ylidene] dimethylammonium chloride (C.I. Basic Blue 26) [with ≥ 0.1% of Michler's ketone (EC No. 202-027-5) or Michler's base (EC No. 202-959-2)] C.I.碱性蓝26 | 2580-56-5 |  | - Used in the formulation of inks, cleaners, and coatings, as well as for dyeing paper, packaging, textiles, plastic products, and other types of articles. It is also used in diagnostic and analytical applications. 主于油墨、清洁剂、涂料的生产；也用于纸张、包装、织物、塑料等产品的着色，以及诊断和分析用途 。 |
| 82 | [4-[4,4'-bis(dimethylamino) benzhydrylidene]cyclohexa-2,5-dien-1-ylidene]dimethylammonium chloride (C.I. Basic Violet 3) [with ≥ 0.1% of Michler's ketone (EC No. 202-027-5) or Michler's base (EC No. 202-959-2)] C.I.碱性紫3 | 548-62-9 |  | - Mainly used for paper colouring and inks supplied in printer cartridges and ball pens. 主要用于纸张着色、印刷墨盒与圆珠 笔墨水 。 - Further uses include staining of dried plants, use as a marker for increasing the visibility of liquids, staining in microbial and clinical laboratories. 未来可能用于干花着色、为染色剂增加液体能见度用于微生物和临床实验室。 |
| 83 | 4,4'-bis(dimethylamino)-4''-(methylamino)trityl alcohol [with ≥ 0.1% of Michler's ketone (EC No. 202-027-5) or Michler's base (EC No. 202-959-2)] 4,4’-二[(二甲氨基) 苯基]-4-甲氨基苯甲醇 | 561-41-1 |  | - Mainly used in the formulation of writing inks and potentially other inks, as well as for dyeing a variety of materials. 主要用于书写墨水的生产；未来可能用 于其他墨水及诸多材料的着色。 |
| 84 | α,α-Bis[4-(dimethylamino)phenyl]-4 (phenylamino)naphthalene-1-methanol (C.I. Solvent Blue 4) [with ≥ 0.1% of Michler's ketone (EC No. 202-027-5) or Michler's base (EC No. 202-959-2)] C.I.溶剂蓝4 | 6786-83-0 |  | - Mainly used in the formulation of printing and writing inks, for dyeing paper and in mixtures such as windscreen washing agents. 主要用于印刷墨水及书写墨水的生产 ； 以及纸张染料，挡风玻璃清洗剂等制剂。 |
| 85 | Bis(pentabromophenyl) ether (DecaBDE) 十溴联苯醚 | 1163-19-5 | 214-604-9 |  |
| 86 | Pentacosafluorotridecanoic acid 全氟十三酸 | 72629-94-8 | 276-745-2 |  |
| 87 | Tricosafluorododecanoic acid 全氟十二烷酸 | 307-55-1 | 206-203-2 |  |
| 88 | Henicosafluoroundecanoic acid 全氟十一烷酸 | 2058-94-8 | 218-165-4 |  |
| 89 | Heptacosafluorotetradecanoic acid 全氟代十四酸 | 376-06-7 | 206-803-4 |  |
| 90 | 4-(1,1,3,3-tetramethylbutyl)phenol, ethoxylated - covering well-defined substances and UVCB substances, polymers and homologues 对特辛基苯酚，乙氧基- 包括界定明确的物质以及UVCB物质、聚合物和同系物 | —— | —— |  |
| 91 | 4-Nonylphenol, branched and linear - substances with a linear and/or branched alkyl chain with a carbon number of 9 covalently bound in position 4 to phenol, covering also UVCB- and well-defined substances which include any of the individual isomers or a combination thereof 分支或线性的壬基酚，含有线性或分支、共价绑定苯酚的9个碳烷基链的物质，包括UVCB物质以及任何含有独立或组合的界定明确的同分异构体的物质 | —— | —— |  |
| 92 | Diazene-1,2-dicarboxamide (C,C'-azodi(formamide)) 偶氮二甲酰胺 | 123-77-3 | 204-650-8 |  |
| 93 | Cyclohexane-1,2-dicarboxylic anhydride (Hexahydrophthalic anhydride - HHPA) 六氢邻苯二甲酸酐 | 85-42-7 | 201-604-9 |  |
| 94 | Hexahydromethylphathalic anhydride、Hexahydro-4-methylphathalic anhydride、Hexahydro-1-methylphathalic anhydride、Hexahydro-3-methylphathalic anhydride 甲基六氢苯酐、4-甲基六氢苯酐、甲基六氢化邻苯二甲酸酐、3-甲基六氢苯二甲酯酐 | 25550-51-0、 19438-60-9、 48122-14-1、 57110-29-9 | 247-094-1、 243-072-0、 256-356-4、 260-566-1 |  |
| 95 | Methoxy acetic acid 甲氧基乙酸 | 625-45-6 | 210-894-6 |  |
| 96 | 1,2-Benzenedicarboxylic acid, dipentylester, branched and linear 支链和直链1，2-苯二羧二戊酯 | 84777-06-0 | 284-032-2 |  |
| 97 | Diisopentylphthalate(DIPP) 邻苯二甲酸二异戊酯 | 605-50-5 | 210-088-4 |  |
| 98 | N-pentyl-isopentylphtalate 邻苯二甲酸正戊基异戊基酯 |  |  |  |
| 99 | 1,2-Diethoxyethane 乙二醇二乙醚 | 629-14-1 | 211-076-1 |  |
| 100 | N,N-dimethylformamide;dimethyl formamide N,N-二甲基甲酰胺 | 68-12-2 | 200-679-5 |  |
| 101 | Dibutyltin dichloride (DBT) 二丁基二氯化锡 | 683-18-1 | 211-670-0 |  |
| 102 | Acetic acid, lead salt, basic 碱式乙酸铅 | 51404-69-4 | 257-175-3 |  |
| 103 | Basic lead carbonate (trilead bis(carbonate)dihydroxide) 碱式碳酸铅 | 1319-46-6 | 215-290-6 |  |
| 104 | lead oxide sulphate 碱式硫酸铅 | 12036-76-9 | 234-853-7 |  |
| 105 | [Phthalato(2-)]dioxotrilead 二盐基邻苯二甲酸铅 | 69011-06-9 | 273-688-5 |  |
| 106 | Dioxobis(stearato)trilead 双(十八酸基)二氧代三铅 | 12578-12-0 | 235-702-8 |  |
| 107 | Fatty acids, C16-18, lead salts C16-18-脂肪酸铅盐 | 91031-62-8 | 292-966-7 |  |
| 108 | Lead bis(tetrafluoroborate) 氟硼酸铅 | 13814-96-5 | 237-486-0 |  |
| 109 | Lead cynamidate 氨基氰铅盐 | 20837-86-9 | 244-073-9 |  |
| 110 | Lead dinitrate 硝酸铅 | 10099-74-8 | 233-245-9 |  |
| 111 | Lead oxide (lead monoxide) 氧化铅 | 1317-36-8 | 215-267-0 |  |
| 112 | Lead tetroxide (orange lead) 四氧化三铅 | 1314-41-6 | 215-235-6 |  |
| 113 | Lead titanium trioxide 钛酸铅 | 12060-00-3 | 235-038-9 |  |
| 114 | Lead Titanium Zirconium Oxide 钛酸铅锆 | 12626-81-2 | 235-727-4 |  |
| 115 | Pentalead tetraoxide sulphate 氧化铅与硫酸铅的复合物 | 12065-90-6 | 235-067-7 |  |
| 116 | Pyrochlore, antimony lead yellow 颜料黄41 | 8012-00-8 | 232-382-1 |  |
| 117 | Silicic acid, barium salt, lead-doped 铅掺杂的硅钡酸盐 | 68784-75-8 | 272-271-5 |  |
| 118 | Silicic acid, lead salt 硅酸铅 | 11120-22-2 | 234-363-3 |  |
| 119 | Sulfurous acid, lead salt, dibasic 亚硫酸铅（II） | 62229-08-7 | 263-467-1 |  |
| 120 | Tetraethyllead 四乙基铅 | 78-00-2 | 201-075-4 |  |
| 121 | Tetralead trioxide sulphate 三碱式硫酸铅 | 12202-17-4 | 235-380-9 |  |
| 122 | Trilead dioxide phosphonate 磷酸氧化铅 | 12141-20-7 | 235-252-2 |  |
| 123 | Furan 呋喃 | 110-00-9 | 203-727-3 |  |
| 124 | Propylene oxide; 1,2-epoxypropane; methyloxirane 环氧丙烷 | 75-56-9 | 200-879-2 |  |
| 125 | Diethyl sulphate 硫酸二乙酯 | 64-67-5 | 200-589-6 |  |
| 126 | Dimethyl sulphate 硫酸二甲酯 | 77-78-1 | 201-058-1 |  |
| 127 | 3-ethyl-2-methyl-2-(3-methylbutyl)-1,3-oxazolidine 3-乙基-2-甲基-2-(3-甲基丁基)-1,3-噁唑烷 | 143860-04-2 | 421-150-7 |  |
| 128 | Dinoseb 地乐酚 | 88-85-7 | 201-861-7 |  |
| 129 | 4,4'-methylenedi-o-toluidine 4,4'-二氨基-3,3'-二甲基二苯甲烷 | 838-88-0 | 212-658-8 |  |
| 130 | 4,4'-oxydianiline and its salts 4,4'-二氨基二苯醚 | 101-80-4 | 202-977-0 |  |
| 131 | 4-Aminoazobenzene; 4-Phenylazoaniline 4-氨基偶氮苯 | 60-9-3 | 200-453-6 |  |
| 132 | 4-methyl-m-phenylenediamine (2,4-toluene-diamine) 2,4-二氨基甲苯 | 95-80-7 | 202-453-1 |  |
| 133 | 6-methoxy-m-toluidine (p-cresidine) 2-甲氧基-5-甲基苯胺 | 120-71-8 | 204-419-1 |  |
| 134 | Biphenyl-4-ylamine 4-氨基联苯 | 92-67-1 | 202-177-1 |  |
| 135 | o-aminoazotoluene 邻氨基偶氮甲苯 | 97-56-3 | 202-591-2 |  |
| 136 | o-Toluidine; 2-Aminotoluene 邻甲基苯胺 | 95-53-4 | 202-429-0 |  |
| 137 | N-methylacetamide N-甲基乙酰胺 | 79-16-3 | 201-182-6 |  |
| 138 | 1-bromopropane 溴代正丙烷 | 106-94-5 | 203-445-0 |  |

Articles suppliers’ obligations

The REACH regulation poses special challenges for suppliers due to the complex supply chain and the numerous ways in which products can be contaminated with SVHCs. The regulation imposes the following obligations on the suppliers:

If an SVHC on the Candidate List is present at >0.1% by weight of the product, the following information must be communicated to the (professional) recipient of the article immediately or within 45 days upon request by a consumer:

- the name of the SVHC as a minimum; and

- sufficient information, available to the supplier, that allows the safe use of the article.

It is important to note that the communication obligation applies as soon as an SVHC is included on the Candidate List.

If an SVHC on the Candidate List is present at >0.1% by weight of the article, and the quantity of the SVHC totals over one tonne per year per (EU) manufacturer/ importer, notification to the ECHA is required.

物品供应商的责任

在复杂的供应链中，产品会在很多途径中会被高度关注物质所污染，因而，REACH法规为物品供应商带来了重大挑战。供应商必须遵守法规下列明的强制要求:

如果产品中含有候选清单中所列出的高度关注物质，且其含量超过0.1% (按重量比) ，供应商则必须即时向下游买家或於收到消费者要求後的45 天内提供以下的相关资讯：

- 至少应提供该物质的名称

- 供应商所拥有对任何使用物品的安全使用指示

要注意：当一项新的高度关注物质被列入候选清单时，供应商对其相关资讯的义务是即时生效。

如被列入候选清单的高度关注物质於成品中之含量超过0.1% (按重量比)，而这物质的全年总数量超出1吨，(欧盟)制造商或进口商需向欧洲化学品管理局(ECHA) 作出通报。